| MATLAB File Name | Sensors Utilized | Acc Placement Aclarations | Subject’s Position During Measurement | Key Findings from Results |
| --- | --- | --- | --- | --- |
| Test1_Seated Folder: Plux_Logger | Plux (ECG + acc) SensorLogger (acc: Headphones) | Plux: Left arm (deltoid) Mobile Phone: Right shoulder | Sitting in a chair |  |
| Test2_Seated Folder: Plux_Logger | Plux (ECG + acc) SensorLogger (acc: Mobile Phone+ Headphones) | Plux: Left arm (deltoid) Mobile Phone: Right shoulder | Sitting in a chair | Mobile Phone Bioplux_Z |
| Test3_Standup Folder: Plux_Logger | Plux (ECG + acc) SensorLogger (acc: Mobile Phone+ Headphones) | Plux: Left arm (deltoid) Mobile Phone: Right shoulder | Stand up | Mobile Phone |
| Test4_Seated Folder: Plux_Logger | Plux (ECG + acc) SensorLogger (acc: Mobile Phone+ Headphones) | Plux: Left clavicle Mobile Phone: Right shoulder | Sitting in a chair | BioPlux |
| Test5_Seated Folder: Plux_Logger | Plux (ECG + acc) SensorLogger (acc: Mobile Phone+ Headphones) | Plux: Left side of the neck (yugular) Mobile Phone: Right shoulder | Sitting in a chair | BioPlux Mobile Phone |
| Test6_Seated Folder: Plux_Logger | Plux (ECG + acc) SensorLogger (acc: Mobile Phone+ Headphones) | Plux: Sternum Mobile Phone: Right shoulder | Sitting in a chair | Mobile Phone |
| Test7_Seated Folder: Plux_Logger | Plux (ECG + acc) SensorLogger (acc: Mobile Phone+ Headphones) | Plux: Sternum Mobile Phone: Right shoulder | Sitting in a chair | Mobile Phone BioPlux |
| Bed_near Folder: Bed | SensorLogger (acc: Mobile Phone) | Near my left side of the chest | Lying on the bed |  |
| Bed_under Folder: Bed | SensorLogger (acc: Mobile Phone) | Under my left side of the chest | Lying on the bed |  |
| Mattress_1 Folder: Bed | SensorLogger (acc: Mobile Phone) | Plux: Sternum Mobile Phone: Near my left side of the chest | Lying on the mattress | BioPlux Mobile Phone |
| Mattress_2 Folder: Bed | SensorLogger (acc: Mobile Phone) | Plux: Sternum Mobile Phone: Near my left side of the chest | Lying on the mattress | Mobile Phone |

POWER SPECTRUM IMPLEMENTATION

- Choosing a specific window

Obtain specific segments of the accelerometer signals (x, y z)

window_scg_x=dataArrayPlux(30000:65000, 3);

- Filtering

Filter the windows using bpFilt, the filter that is the one configured for the accelerometer signals up in the code (band-pass filter from 1-14Hz, 4th order and the corresponding sample frequency).

scg_x_filtered = filtfilt(bpFilt, window_scg_x);

- Module calculation

Obtain the magnitude of the filtered signals (x, y z).

mod=sqrt(scg_x_filtered.^2 + scg_y_filtered.^2 + scg_z_filtered.^2);

- Fast Fourier Transform

Transform the filtered signals and the module to the frequency domain, best visualization of the frequency components of the signal.

fft_scg_x = fft(scg_x_filtered);

- Unilateral Spectrum (Real Signal)

Only compute the positive values of the FFT (floor(L/2)+1). Calculation of the magnitude of the FFT (abs) and normalize it by L, the length of the signal.

L = length(window_scg_x);

P1_x = abs(fft_scg_x(1:floor(L/2)+1))/L;

- Frequency vector associated with the spectral power

This help to associate the power values back to their respective frequencies.

f_one_sided = Fs*(0:(floor(L/2)))/L;

- Only work with frequencies that are less 30 Hz

Index that contains only frequencies that less or equal than 30 Hz (x axes in the graphic)

idx = f_one_sided <= 30;

- Consider the band of 0.8-1.8 Hz, where the SCG/BCG signal should appear

idx_band = f_one_sided >= 0.8 & f_one_sided <= 1.8;

- Total Spectral Power Calculation

Calculated by summing the squares of the magnitudes of the FFT calculation, we get the total energy in the signal.

total_power_x = sum(P1_x.^2);

- Spectral Power in the Band [0.8-1.8 Hz] Calculation

Summing up the power in just the band using the index that select this correct frequency range.

band_power_x = sum(P1_x(idx_band).^2);

- Divide the Band Power with the Total Power (Ratios)

This help as to visualize and quantify how much of the total spectrum power is concentrated in the band of interest. Show the values in the command window.

ratio_x = band_power_x / total_power_x;

- Plot the Power Spectrum in the frequency of interest (<30 Hz)

The x axis is represented by f_one_sided(idx).

The y axis is represented by P1_x(idx).

plot(f_one_sided(idx), P1_x(idx));

Ratio Calculation Mobile Phone:

Ratio X-axis: 0.47688

Ratio Y-axis: 0.19674

Ratio Z-axis: 0.22438

Ratio of the module: 0.055021

Ratio Calculation Headphones:

Ratio X-axis: 0.034749

Ratio Y-axis: 0.034394

Ratio Z-axis: 0.011299

Ratio of the module: 0.0060535

Ratio Calculation BioPlux:

Ratio X-axis: 0.092575

Ratio Y-axis: 0.22375

Ratio Z-axis: 0.11846

Ratio of the module: 0.019672

Test 2

Test 3

Test 4

Test_5

Test_6

Test 7

Mattress_1

Mattress_2